Table 2-5Summary  of laboratory methods for the analysis of organic compounds in sediment samples.  (All concentrations were reported on a dry weight basis in ng/g.)

| Analyte Group | Extraction Methods and Reference | Cleanup Method and Reference | Fractionation Method and Reference | Instrumental Method and Reference | Surrogate Compounds | Internal Standards |
| --- | --- | --- | --- | --- | --- | --- |
| PAHsa | Sequential solvent extraction with sonication; followed by drying over NO2SO4 and concentration by Kuderna-Danish evaporation (SOP ADL-2819) | Alumina-preparative chromatography column (SOP ADL-2821) | High Performance Liquid Chromatography (SOP ADL-2821) | GC/MS, Selected Ion Monitoring Mode (SOP ADL-2827) | Acenaphthene-d10 Phenanthrene-d10 Benzo(a)pyrene-d12 Napthalene-d8 | Fluorene-d10 Chrysene-d12 |
| PCBs, Chlorinated Pesticidesa | Same | Same | Same | Single-column gas chromatography-electron capture detection (GC-ECD) | 4,4'-dibromoocta-flurobiphenyl (DBOFB) PCB congeners 81,103,198 | Tetrachloro-m-xylene (TCMX) |

aRefer to Tables 2-2 and 2-3 for lists of individual target PAHs and PCBs/Pesticides, respectively.