| Parameter | Matrix | Units | Methodology | Reference | Performing Laboratory | Assessment Use |
| --- | --- | --- | --- | --- | --- | --- |
| Moisture Content | Sediment | Weight % | Dry & weigh | - | ADL | Report Contaminant concentrations on a dry weight basis. |
| Total Organic Carbon (TOC) | Sediment | Weight % | Acid treatment to remove inorganic carbon followed by combustion at 900C in Leco carbon analyzer | SOP ADL-2829 | Global Geochemistryb | 1.Measure of organic carbon loading in sediments. 2.Used to normalize organic chemical parameters. |
| Eha | Sediment | mV | Orion model 9678 BN redox probe | - | ADL | Assess extent of anoxia in subsurface sediment layers. |
| Grain size | Sediment | Weight % gravel, sand, silt, clay | Sieve and pipette | Folk (1974) | GEO/PLANc | 1.Provides information on particle size distribution of sediments and nature of sedimentary environment. 2.Used as a normalization factor for organic compounds and metals. 3.Influence on infaunal distributions. |
| Salinitya | Near-bottom water | ppt | Horiba model U10 water-quality probe system | - | ADL | Influence on biological/ toxicological responses. |
| Temperaturea | Near-bottom water | C | Horiba model U10 water-quality probe system | - | ADL | Influence on biological/ toxicological responses. |
| pHa | Near-bottom water | - | Horiba model U10 water-quality probe system | - | ADL | Influence on biological/ toxicological responses. |
| Dissolved oxygen(DO)a | Near-bottom water | mg/L (ppm) | Horiba model U10 water-quality probe system | - | ADL | Influence on biological/ toxicological responses. |
| Water Deptha | - | m | Fathometer | - | ADL | Influence on biotic and abiotic variables. |
| Temperature | Pore water | C | YSI model 59 DO/ temperature meter | - | Corpus Christi State Univ. | Influence on biological/ toxicological responses. |
| Salinity | Pore water | ppt | Reichert temperature-compensated refractometer | - | Corpus Christi State Univ. | Influence on biological/ toxicological responses. |
| pH | Pore water | pH units | Orion model 290A pH meter | - | Corpus Christi State Univ. | Influence on biological/ toxicological responses. |
| DO | Pore water | mg/L and % saturation | YSI model 59 DO meter | - | Corpus Christi State Univ. | Influence on biological/ toxicological responses. |
| Sulfide | Pore water | mg/L | Orion (model 94-12) ion- selective electrode in conjunction with Orion (model 290A) meter | - | Corpus Christi State Univ. | Influence on biological/ toxicological responses. |
| Ammonia (total ammonia and unionized) | Pore water | mg/L | Orion (model 95-12) ion- selective electrode in conjunction with Orion (model 290A) meter | - | Corpus Christi State Univ. | Influence on biological/ toxicological responses. |

aIn-situ field measurement

a     aIn-situ field measurement

bGlobal Geochemistry, Canoga Park, CA

cGEO/PLAN Associates, Hingham, MA