Table 3-14.Summary of sediment quality triad results (adapted from Chapman et al. 1990, and Carr 1993).

| Chemistrya | Toxicityb | Benthosc | Stations | Possible Conclusions |
| --- | --- | --- | --- | --- |
| + | + | + | 1,3,6,7,10,11 | Evidence of environmental impacts caused by measured contaminants or other measured adverse conditions (such as high ammonia or sulfide levels). |
| - | - | - | none | No evidence of environmental degradation. |
| + | - | - | none | Measured contaminants are present at potentially toxic levels but not bioavailable. |
| - | + | - | none | Unmeasured contaminants or adverse conditions exist with the potential to cause degradation, but no clear benthic response. |
| - | - | + | none | Degraded benthos not due to presence of measured contaminants or conditions. |
| + | + | - | 4,5,8,9,12 | Measured contaminants or conditions have the potential to cause degradation, but no clear benthic response. |
| - | + | + | 2 | Unmeasured contaminants or conditions are causing degradation. |
| + | - | + | none | Contaminants are not bioavailable or benthic response not due to chemistry. |

a	(+)Indicates one or more chemical contaminants exceeded NOEL or ER-L values (high ammonia or sulfide concentrations may have occurred as well).

(-)Indicates no exceedance for any measured contaminant.

b	(+)Indicates one or more toxicity endpoints were significantly different than control.

(-)	Indicates no apparent toxicity effect.

c	(+)Indicates low species richness (total number of species from combined replicates within a station  50).

(-)Indicates no apparent adverse benthic effect.